ExactCAD Expedition Hotkeys

| ` | select entire net |
| --- | --- |
| CTRL+’ | lock selected |
| CTRL+1 | select entire net and lock it |
| 1 | Set top layer active |
| 2 | Set bottom layer active |
| Alt+1 | Set Display Scheme 1 |
| Alt+2 | Set Display Scheme 2 |
| 3 | unlock and/or unfix selected objects |
| CTRL + 3 | Select entire net and unlock it |
| 4 | diff pair 45 pad exit |
| ALT + 4 | Diff pair 45 pad exit the opposite direction |
| CTRL+3 | select entire net and unlock it |
| CTRL + F1 | Unhilight all |
| ALT + F1 | Select Entire Net and Highlight it |
| A or a | Align pins, vias and parts, fill pad with vias |
| Alt a or Alt A | Set Selection Mode (this is the Arrow toolbar button to the left of the Place mode button) |
| Ctrl+Shift+a or A | Toggle Conductive Areas |
| C or c | Set Placement mode (c for component) |
| ALT + c | Change text and graphics userlayer all at once |
| D or d | Set Draw mode |
| ALT + d | Toggle Dim Mode between 100% and 40% |
| E or e | Toggle edit layer only |
| ALT + e | Edit Text (needs edit_text.exe to work) |
| G or g | Clump parts into a group |
| CTRL+g | change via to GND Net |
| J or j | Join Lines (works in draw mode on userlayer graphics and traces) (needs join_lines.exe to work) |
| L or l | Lock selected components |
| M or m | Move with Basepoint |
| Alt m | Move selected component to centroid of selected userlayer objects. |
| N or n | Copy net name to clipboard |
| ALT + n | Change net to selected pin net |
| P or p | Execute RF/Power Pin Exit with current settings from configuration tool |
| Alt+P or Alt+p | Toggle Pins on/off |
| SHIFT + p | place plane shape (select a via or pin on the layer you want to use to draw the plane shape first. The via or pin sets the net of the plane shape. |
| R or r | Set Route mode |
| CTRL + r | Rotate selected part 45 (one part only) |
| SHIFT + r | Rotate selected parts 45 (more than one part) |
| CTRL+SHIFT + r | Rotate selected parts by a user-input angle |
| S or s | Toggle hover snap in Xpedition |
| ALT + s | Swap the locations of 2 selected parts |
| CTRL+t | set selected trace width equal to narrow pad dimension (selected pad) |
| ALT+t | set selected trace width equal to wide pad dimension |
| T or t | trim traces |
| SHIFT + t | Toggle traces on / off |
| U or u | Unlock selected components |
| V or v | Toggle via visibility on/off |
| W or w | Get active route width |
| Shift+w | set trace width to pad width on the side you’re exiting |
| X or x | Toggle DRC on/off |